# Index

## Device Specs

## Network Diagrams

## TV Outlets

## Wiring Logs

## Technical Logs

## As Built Drawing

# Device Specs

| Device |  | Link |
| --- | --- | --- |
| Allied Telesis Switch AT-GS970M/28PS Allied Telesis Switch AT-GS950- 24-10 |  | https://www.alliedtelesis.com/sites/default/files/documents/datasheets/ati-gs970mseries-ds.pdf https://ftp3.syscom.mx/usuarios/adiaz/AlliedTelesis/GS950/gs950_ds_revl_reduced.pdf |
| Unifi Access Point U6-LR Netgear Wifi Router R7000 TrippLite Smart 750 USB Unifi Cloud Key Truaudio Sound Bar SB-SLIM44P |  | http://dl-origin.ubnt.com/ds/u6-lr_ds.pdf https://www.netgear.es/images/datasheet/networking/wifirouter/R7000.pdf. https://cdn.cyberpuerta.mx/storage/articles/multimedia/smart750usb.pdf https://dl.ubnt.com/datasheets/unifi/UniFi_Cloud_Key_G2_Plus_DS.pdf https://vw-truaudio.storage.googleapis.com/dealer_resources/OnesheetSB-SLIM-44P.pdff |
| Truaudio Speakers CSUB-10 |  | https://vw-truaudio.storage.googleapis.com/dealer_resources/OnesheetCSUBc.pdf |
| Phantom Speakers PG-6 |  | https://vw-truaudio.storage.googleapis.com/dealer_resources/Phantom-onesheet_11-4.pdf |
| Von Duprin 6400 |  | https://us.allegion.com/content/dam/allegion-us-2/web-documents-2/DataSheet/Von_Duprin_ES-6400_Series_Modular_Strike_Data_Sheet_109011.pdf |
| Net Smoke Detector S3004PWBUS Ring Video Doorbell 8VR1E7-0EN0 |  | https://images.thdstatic.com/catalog/pdfImages/97/97ea2318-5551-4fa0-9021-5a793a883d90.pdf https://www.supercircuits.com/media/docs/8vr1e7-0en0-datasheet-dlr.pdf |

# Network Diagrams

# TV Outlets

# Wiring Logs

Wiring Logs – Estero Villa 13A

| AP (Access Points) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ID | Cable | Color | Brand | From | To |
| AP01 | UTP-CAT6 | Blue | Leviton | Rack | TV Living Room |
| AP02 | UTP-CAT6 | Blue | Leviton | Rack | TV Terrace |
| AP03 | UTP-CAT6 | Blue | Leviton | Rack | Terrace |
| AP04 | UTP-CAT6 | Blue | Leviton | Rack | TV Master Bed |
| AP05 | UTP-CAT6 | Blue | Leviton | Rack | TV Bedroom 2 |
| AP06 | UTP-CAT6 | Blue | Leviton | Rack | TV Bedroom 3 |
| AP07 | UTP-CAT6 | Blue | Leviton | Rack | TV Bedroom 4 |
| AP08 | UTP-CAT6 | Blue | Leviton | Rack | TV Bedroom 5 |
| Data |  |  |  |  |  |
| ID | Cable | Color | Brand | From | To |
| DAT01 | UTP-CAT 6 | Blue | Leviton | Rack | Living Room |
| DAT02 | UTP-CAT 6 | Blue | Leviton | Rack | TV Living Room |
| DAT03 | UTP-CAT 6 | Blue | Leviton | Rack | TV Terrace |
| DAT04 | UTP-CAT 6 | Blue | Leviton | Rack | Master Bed |
| DAT05 | UTP-CAT 6 | Blue | Leviton | Rack | TV Master Bed |
| DAT06 | UTP-CAT 6 | Blue | Leviton | Rack | Master Bath |
| DAT07 | UTP-CAT 6 | Blue | Leviton | Rack | Bedroom 2 |
| DAT08 | UTP-CAT 6 | Blue | Leviton | Rack | TV Bedroom 2 |
| DAT09 | UTP-CAT 6 | Blue | Leviton | Rack | Bedroom 3 |
| DAT10 | UTP-CAT 6 | Blue | Leviton | Rack | TV Bedroom 3 |
| DAT11 | UTP-CAT 6 | Blue | Leviton | Rack | Bedroom 4 |
| DAT12 | UTP-CAT 6 | Blue | Leviton | Rack | TV Bedroom 4 |
| DAT13 | UTP-CAT 6 | Blue | Leviton | Rack | Bedroom 5 |
| DAT14 | UTP-CAT 6 | Blue | Leviton | Rack | TV Bedroom 5 |
| DAT15 | UTP-CAT 6 | Blue | Leviton | Rack | Laundry |
| DAT16 | UTP-CAT 6 | Blue | Leviton | Rack | Lutron |
| DAT17 | UTP-CAT 6 | Blue | Leviton | Rack | Lutron |
| DAT18 | UTP-CAT 6 | Blue | Leviton | Rack | Cuarto de maquinas |
| Iptv |  |  |  |  |  |
| ID | Cable | Color | Brand | From | To |
| TV01 | UTP-CAT6 | Yellow | Leviton | Rack | TV Living Room |
| TV02 | UTP-CAT6 | Yellow | Leviton | Rack | TV Terrace |
| TV03 | UTP-CAT6 | Yellow | Leviton | Rack | TV Master Bed |
| TV04 | UTP-CAT6 | Yellow | Leviton | Rack | TV Bedroom 2 |
| TV05 | UTP-CAT6 | Yellow | Leviton | Rack | TV Bedroom 3 |
| TV06 | UTP-CAT6 | Yellow | Leviton | Rack | TV Bedroom 4 |
| TV07 | UTP-CAT6 | Yellow | Leviton | Rack | TV Bedroom 5 |
| Telephone |  |  |  |  |  |
| ID | Cable | Color | Brand | From | To |
| TEL01 | UTP-CAT 6 | Gray | Leviton | Rack | Living room |
| TEL02 | UTP-CAT 6 | Gray | Leviton | Rack | Master Bed |
| TEL03 | UTP-CAT 6 | Gray | Leviton | Rack | Bedroom 2 |
| TEL04 | UTP-CAT 6 | Gray | Leviton | Rack | Bedroom 3 |
| TEL05 | UTP-CAT 6 | Gray | Leviton | Rack | Bedroom 4 |
| TEL06 | UTP-CAT 6 | Gray | Leviton | Rack | Bedroom 5 |
| TEL07 | UTP-CAT 6 | Gray | Leviton | Rack | Laundry |

| Tv Area - Audio Return |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ID | Cable | Color | Brand | From | To |
| TVOUT01 | MIC | Black | Belden | Rack | TV Living Room |
| TVOUT02 | MIC | Black | Belden | Rack | TV Terrace |
| TVOUT03 | MIC | Black | Belden | Rack | TV Master Bed |
| TVOUT04 | MIC | Black | Belden | Rack | TV Bedroom 2 |
| TVOUT05 | MIC | Black | Belden | Rack | TV Bedroom 3 |
| TVOUT06 | MIC | Black | Belden | Rack | TV Bedroom 4 |
| TVOUT07 | MIC | Black | Belden | Rack | TV Bedroom 5 |

| Tv Area - Spare |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ID | Cable | Color | Brand | From | To |
| TVS01 | UTP-CAT 6 | Blue | Leviton | Rack | TV Living Room |
| TVS02 | UTP-CAT 6 | Blue | Leviton | Rack | TV Terrace |
| TVS03 | UTP-CAT 6 | Blue | Leviton | Rack | TV Master Bed |
| TVS04 | UTP-CAT 6 | Blue | Leviton | Rack | TV Bedroom 2 |
| TVS05 | UTP-CAT 6 | Blue | Leviton | Rack | TV Bedroom 3 |
| TVS06 | UTP-CAT 6 | Blue | Leviton | Rack | TV Bedroom 4 |
| TVS07 | UTP-CAT 6 | Blue | Leviton | Rack | TV Bedroom 5 |
| Site - Communication Panel |  |  |  |  |  |
| ID | Cable | Color | Brand | From | To |
| Data ONU | UTP-CAT6 | Blue | Leviton | Rack | ONU-CP |
| Tel ONU | UTP-CAT6 | Gray | Leviton | Rack | ONU-CP |
| Tv ONU | UTP-CAT6 | Yellow | Leviton | Rack | ONU-CP |
| VPN ONU | UTP-CAT6 | Yellow | Leviton | Rack | ONU-CP |

| Other Services |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ID | Cable | Color | Brand | From | To |
| Doorphone | UTP-CAT6 | Gray | Leviton | Rack | Door Phone Main Entry |
| Stricker Doorphone | UTP-CAT6 | Gray | Leviton | Rack | Stricker Door Phone Main Entry |

| Audio Wiring |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ID | Cable | Color | Brand | From | To |
| Kitchen R | 16/2 | White | UC | Site -Rack Equipment | Kitchen R |
| Kitchen L | 16/2 | White | UC | Site -Rack Equipment | Kitchen L |
| Dining R | 16/2 | White | UC | Site -Rack Equipment | Dining L |
| Dining L | 16/2 | White | UC | Site -Rack Equipment | Dining R |
| Living Front R | 16/2 | White | UC | Site -Rack Equipment | Living room R |
| Living Front L | 16/2 | White | UC | Site -Rack Equipment | Living room L |
| Terrace R | 16/2 | White | UC | Site -Rack Equipment | Terrace R |
| Terrace L | 16/2 | White | UC | Site -Rack Equipment | Terrace L |
| Terrace R | 16/2 | White | UC | Site -Rack Equipment | Terrace R |
| Terrace L | 16/2 | White | UC | Site -Rack Equipment | Terrace L |
| Terrace R | 16/2 | White | UC | Site -Rack Equipment | Terrace R |
| Terrace L | 16/2 | White | UC | Site -Rack Equipment | Terrace L |
| Master Bed R | 16/2 | White | UC | Site -Rack Equipment | Master Bed R |
| Master Bed L | 16/2 | White | UC | Site -Rack Equipment | Master Bed L |
| Master Bath D1 | 16/2 | White | UC | Site -Rack Equipment | Master Bath Dual 1 |
| Master Bath D2 | 16/2 | White | UC | Site -Rack Equipment | Master Bath Dual 2 |
| Bedroom 2 R | 16/2 | White | UC | Site -Rack Equipment | Bedroom 2 R |
| Bedrooom 2 L | 16/2 | White | UC | Site -Rack Equipment | Bedrooom 2 L |
| Bedroom 3 R | 16/2 | White | UC | Site -Rack Equipment | Bedroom 3 R |
| Bedrooom 3 L | 16/2 | White | UC | Site -Rack Equipment | Bedrooom 3 L |
| Bedroom 4 R | 16/2 | White | UC | Site -Rack Equipment | Bedroom 4 R |
| Bedrooom 4 L | 16/2 | White | UC | Site -Rack Equipment | Bedrooom 4 L |
| Bedroom 5 R | 16/2 | White | UC | Site -Rack Equipment | Bedroom 5 R |
| Bedrooom 5 L | 16/2 | White | UC | Site -Rack Equipment | Bedrooom 5 L |

# Technical Logs

As Built Drawing

Waicura Technologies S.A. de C.V.

Project Manager: Jonathan Lopez.

Project Leader: Luciano Gutiérrez.